Introduction: Sorafenib is a multikinase inhibitor that was approved by the FDA in 2007 for the treatment of hepatocellular carcinoma (HCC). Since then, it has quickly become the standard of care for patients with advanced stage disease.  However, patient tolerance is an issue due to the side effects resulting from systemic delivery.  To avoid systemic delivery of chemotherapies for HCC, a procedure called transarterial chemoembolization (TACE) has been developed to selectively place a catheter in tumor feeding arteries to locally deliver the chemotherapy.  This is then followed by or combined with embolic agents to contain the chemotherapy and to further starve the tumor from nutrients and oxygen in the blood.  Thus, poly(lactide-co-glycolide) (PLG) microspheres have been developed that can encapsulate sorafenib for local delivery to liver tumors via TACE as well as co-encapsulating iron oxide nanoparticles to enable MRI-monitoring of the delivery.  Specifically, the purpose of this study aims to use MRI to correlate to microsphere concentrations at the tumor to determine estimated sorafenib doses delivered.

Methods: PLG sorafenib iron oxide microspheres were fabricated via a double emulsion/solvent evaporation method.  The microspheres were then characterized for microsphere size, sorafenib and iron oxide loading and release, as well as MRI contrast effect properties in vitro (7T MRI, Bruker Clinscan, Billerica, MA, USA).  From these in vitro MRI studies, the microsphere concentrations were plotted to correlate against changes in MRI signal changes.  Afterwards, liver tumors were inoculated in 4  New Zealand White Rabbits in which a VX2 tumor fragment was injected through a biopsy needle under ultrasound guidance.  The tumor was then allowed to grow to roughly 1 cm in diameter after 2-3 week after which the rabbits underwent TACE procedures and microsphere infusions (50 mg).  The rabbits were also scanned pre-and post procedurally at the same 7T MRI Bruker Clinscan to compare tumor signals before and after microsphere infusion with T2 and T2* map protocols.   Afterwards, the tumor T2 and T2* values were extracted from the MRI images via MATLAB analyses to determine changes in the T2 and T2*.  These changes were then used to estimate the microsphere concentrations at the tumor using the correlations previously obtained following the in vitro MRI studies.  The rabbits were then survived for 24 hours before sacrificing and harvesting the tissue for Prussian blue staining for iron.

Results: The resulting microspheres were of an average diameter of 13 microns.  Loading and release studies indicated that the weight percentage of sorafenib and iron oxide in the microspheres were 8.8% and 0.89%, respectively and that 21% and 28% of the loaded sorafenib and iron oxide released in 72 hours, respectively.   In vitro MRI studies showed that microsphere concentration was correlated to 1/T2* via the equation: (1/T2*) = 0.0205(Microsphere Concentration) + 0.1262.  For subsequent in vivo studies, MRI indicated that the microspheres primarily deposited in the tumor periphery and the estimated microsphere concentrations were on average 45 mg/mL in those regions.  After the rabbits were sacrificed, Prussian blue staining confirmed proper tumor targeting and that primary microsphere deposition was at the tumor periphery.

Conclusion: The developed PLG sorafenib iron oxide microspheres allow for targeted delivery of sorafenib to liver tumors with the advantage of MRI monitoring and proper estimations of the delivered dose.  Further studies are evaluating biomarkers for tumor responses and possible correlations of the tumor responses to the microsphere doses that were effectively delivered.