Tuning Cellulose Pyrolysis Chemistry: Selective Decarbonylation via Catalyst-Impregnated Pyrolysis

Alex D. Paulsen, Matthew S. Mettler, Dionisios G. Vlachos, and Paul J. Dauenhauer

Abstract: Production of bio-oils and biofuels from lignocellulosic biomass relies on the chemistry of high temperature cellulose pyrolysis.1 Cellulose is known to react through a short-lived liquid intermediate2 during pyrolysis to produce a variety of condensable organics.3-5 However, utilization of biomass as a fossil fuel replacement is hindered by the instability of bio-oil under storage conditions, likely caused by polymerization of aldehydes present in bio-oil. Therefore, we examine cellulose impregnation of supported metal catalysts to tune the chemistry within the liquid intermediate and reduce the production of aldehydes. We show that using a carbon-supported metal catalysts significantly reduce the yield of aldehydic products by converting hydroxymethylfurfural and furfural to carbon monoxide and deoxygenated furans (methyl furan, furan, dimethylfuran) while maintaining the overall bio-oil yield.

1.	Paulsen, A. D., Hough, B. R., Williams, C. L., Teixeira, A. R., Schwartz, D. T., Pfaendtner, J., &amp; Dauenhauer, P. J., Fast Pyrolysis of Wood for Biofuels: Spatiotemporally Resolved Diffuse Reflectance In situ Spectroscopy of Particles. ChemSusChem 2014, 7, (3), 765-776.

2.	Dauenhauer, P. J.; Colby, J. L.; Balonek, C. M.; Suszynski, W. J.; Schmidt, L. D., Reactive boiling of cellulose for integrated catalysis through an intermediate liquid. Green Chemistry 2009, 11, (10), 1555-1561.

3.	Mettler, M. S.; Mushrif, S. H.; Paulsen, A. D.; Javadekar, A. D.; Vlachos, D. G.; Dauenhauer, P. J., Revealing pyrolysis chemistry for biofuels production: Conversion of cellulose to furans and small oxygenates. Energy & Environmental Science 2012, 5, (1), 5414-5424.

4.	Mettler, M. S.; Paulsen, A. D.; Vlachos, D. G.; Dauenhauer, P. J., The chain length effect in pyrolysis: bridging the gap between glucose and cellulose. Green Chemistry 2012, 14, 1284-1288.

5.	Paulsen, A. D., Mettler, M. S., Vlachos, D. G., &amp; Dauenhauer, P. J., The Role of Sample Dimension and Temperature in Cellulose Pyrolysis. Energy & Fuels 2013.