Kinetics of Jatropha oil methanolysis

Mario Andrés Noriega Valencia1, Paulo César Narváez Rincón1, Christoph Heinz2, Nevardo Bello Yaya1, Juan Guillermo Cadavid Estrada1

1.Grupo de Procesos Químicos y Bioquímicos, Departamento de Ingeniería Química. Facultad de Ingeniería, Universidad Nacional de Colombia, Bogotá, Colombia.

2 Technische Universität München, Chemistry Department, Lichtenbergstraße 4, D-85748 Garching b. München, Germany.

*pcnarvaezr@unal.edu.co

Abstract

Biodiesel is currently produced by methanolysis of vegetable oils such as rapeseed, palm and soybean oils. However, Jatropha oil is considered one of the most promising sources for producing biodiesel in Asia, Europe and Africa, The knowledge of the kinetics of his methanolysis reaction is extremely important because it enables the design of  reactors with suitable dimensions and the operation  in optimal conditions. In this work the kinetics of Jatropha oil methanolysis was studied. The effect of temperature, catalyst concentration and methanol to oil molar ratio on Jatropha oil methanolysis were studied in the range from 40 to 60 °C, 0.2 to 1wt% (NaOH) and 3 to 12, respectively. A 100% conversion of Jatropha oil with a 97% yield to fatty acid methyl esters (FAME) were obtained after 60 minutes of reaction at 60°C, with 0.6wt% of NaOH, and methanol to oil molar ratio of 6:1. Catalyst concentration increments between 0.2wt% and 0.6wt% had a positive effect on the reaction rate and on the yield to FAME. However, the 0.2wt% catalyst concentration did not show activity at all reaction temperatures. A second order kinetic model that includes the effect of temperature and catalyst concentration on reaction rate describes adequately Jatropha oil methanolysis, as validated by the Fisher–Snedecor test of unbiased variances. Kinetic model parameters of Jatropha oil methanolysis, Palm oil methanolysis and Palm oil ethanolysis were compared and showed similar values and behavior.

Keywords: Jatropha oil, Palm oil, Methanolysis, Ethanolysis Biodiesel, Kinetic model, Mathematic model