ID: 378044

Password: 248756

AIChE – Biomaterials – 08B15 Biomaterials: Faculty Candidates I

Title: Self-Assembled Hydrogels Utilizing Polymer-Nanoparticle Interactions

Authors: Mark W. Tibbitt, Eric A. Appel, and Robert Langer

Hydrogels are an important class of biomaterial that have received significant attention for controlled drug-delivery applications on account of their similarity to soft biological tissue and highly tunable mechanical properties.1 Shear-thinning hydrogels are water-swollen dynamically cross-linked polymer networks exhibiting viscous flow under shear stress (shear-thinning) and rapid recovery of mechanical properties when the applied stress is relaxed (self-healing).2 These properties afford minimally invasive implantation in vivo though direct injection or catheter delivery to tissues, contributing to a rapid gain in interest in their application for drug delivery and tissue engineering.3 Herein, we report the preparation and application of shear-thinning, injectable hydrogels driven by non-covalent interactions between hydrophobically-modified biopolymers (BPs) and drug-loaded biodegradable nanoparticles (NPs) comprised of poly(ethylene glycol)-block-poly(lactic acid) (PEG-block-PLA). This materials approach builds upon the recent paradigm of NPs as molecular binders in polymeric systems.4 The transient and reversible interactions between BPs and NPs enable flow under applied shear stress, followed by rapid self-healing when the stress is relaxed, demonstrating the shear-thinning and injectable nature of the material. The inclusion of PEG-block-PLA NPs facilitates a high-loading content of hydrophobic molecules within the hydrogel, while a second, hydrophilic molecule, can be loaded simultaneously into the aqueous phase of the gel. Delivery of the hydrophilic molecule is controlled by Fickian diffusion through the gel whereas erosion-based release from the gel surface governs delivery of the hydrophobic molecule. In this manner, multiple therapeutic compounds can be delivered with differential release curves. Additionally, the biocompatible, shear-thinning hydrogels can be implanted into the subcutaneous space of mice for controlled release of therapeutics in vivo or as a cell carrier for regenerative medicine applications. Overall, this presentation will demonstrate the facile synthesis of an injectable hydrogel affording controlled delivery of therapeutics with minimally invasive application.

References

- Hoffman AS Adv Drug Delivery Rev 54 (2002) 3-12

- Appel EA et al. Chem Soc Rev 107 (2010) 18392-18397

- Guvendiren M et al. Soft Matter 8 (2012) 260-272

- Rose S et al. Nature 505 (2014) 382-385