DYNAMIC PROCESS MODELING OF A SUSTAINABLE COAL, WIND, AND SMALL MODULAR REACTOR HYBRID ENERGY SYSTEM

Kyle L. Buchheit and Joseph D. Smith, Ph.D.

Missouri University of Science and Technology, Rolla Missouri

ABSTRACT

The availability of cheap electricity is one of the biggest factors for improving quality of life.  With the debate on the effects of carbon dioxide emissions continuing, several countries have either implemented or are considering the reduction of emissions through various economic means.  The inclusion of a monetary penalty on carbon emissions would increase the prices of electricity produced by carbon-based sources. The push for large-scale renewable sources of energy has met problems with regards to energy storage and availability.  The proposed coal, wind and nuclear hybrid energy system would combine a renewable energy source, wind, with traditional and stable energy sources, coal and nuclear, to create an integrated and sustainable system.  Instead of a light water reactor, a next generation small modular nuclear reactor will be evaluated.  The coal system will use a pressurized circulating fluidized bed system, which can not only take coal, but also biomass as a carbon feedstock.  This system also employs a high temperature steam co-electrolysis unit for the utilization of carbon dioxide emissions for the production of synthetic gas which can be used in the production of transportation fuels or chemicals.

A rigorous dynamic process model of the system will be constructed.  This model will allow for proper scaling of each individual component in the overall hybrid system along with the dynamic characteristics inherent in a fluctuating wind system.  Increased integration of the water and heat cycles will be evaluated for improved efficiency.  A detailed Aspen Plus user model will be utilized to demonstrate the feasibility of the main hybrid energy system components.  Aspen Dynamics and MatLAB / Simulink modeling will integrate the main process model with the dynamic wind model to look at overall sustainability, power generation, and chemical production.