AICHE Abstract

Engineering of A. ferrooxidans for Application in a Novel Non-photosynthetic Electrofuel Production Platform

Xiaozheng Li, Chemical Engineering, Columbia University, New York, NY

Sudipta Majumdar, Chemical Engineering, Columbia University, New York, NY

Tim Kernan, Chemical Engineering, Columbia University, New York, NY

Roel Mercado, Chemical Engineering, Columbia University, New York, NY

Alan West, Chemical Engineering, Columbia University, New York, NY

Scott Banta, Chemical Engineering, Columbia University, New York, NY

Electricity generated from renewable sources has become increasingly significant in modern society. In 2013 nearly 10% of U.S. energy consumed was produced from renewable sources. However one critical challenge for renewables is the storage of generated electricity. Lithium ion batteries currently provide energy density up to 1 MJ/kg. However the chemical bonds of hydrocarbons represent a more convenient and dense form of energy storage; for example, gasoline has an energy density of 46 MJ/kg. We are developing a novel carbon-neutral electrofuel system, where a genetically modified bacterium, A. ferrooxidans, converts atmospheric CO2 into liquid fuels such as heptadecane and isobutyrate, while obtaining energy via the oxidation of electrochemically regenerated Fe2+.

We studied the relationship between pH and growth kinetics in chemostat cultures of wild type A. ferrooxidans cells. We found out that in precipitate-free cultures at low Fe2+ concentrations, the maximum specific growth rate and cell yield increased while maintenance coefficient decreased with increasing pH, indicating improved efficiency at higher pH. However, this trend was interrupted at higher Fe2+ concentrations due to formation of undesirable Fe3+ precipitates. Fe3+ precipitation was successfully resolved by introducing the iron chelating agent, citrate. Citrate not only solubilized Fe3+ but also increased maximum cell yield. We built a prototype integrated system consisting of a chemostat and an electrochemical cell. Wild type cells grew on electrochemically reduced Fe2+ over a two-week time and the system produced 5.2 g cells/kWh.

Two strains of A. ferrooxidans have been successfully transformed with two different biosynthetic pathways produced isobutyrate and heptadecane respectively. In the first strain, 2-ketoacid decarboxylase (KDC) was expressed to convert α-ketoisovalerate into isobutyraldehyde which was then spontaneously oxidized into isobutyrate in the medium. In the second strain, we introduced a pathway to convert long chain fatty acids into alkanes and alkenes. Isobutyrate producing strain was extensively characterized. In the original culture conditions, the strain produced 0.4 ppm (63 µM/M Fe2+) at 0.87% of maximum efficiency. We increased these metrics by optimizing batch growth conditions i.e. initial cell density, initial Fe2+ concentration and iron chelators and MgSO4 concentrations, to 5.8 ppm (377 µM/M Fe2+) at 5.2% of maximum efficiency.  We are currently improving and further characterizing the genetically modified A. ferrooxidans strains in chemostat cultures and in the integrated system.