ABSTRACT

Self-heat recuperation system by electrocaloric effect

Toshihiro Kaseda, Yasuki Kansha, Masanori Ishizuka, Yui, Kotani, Renaldo Rasfuldi, Atsushi Tsutsumi *

Collaborative Research Center for Energy Engineering, Institute of Industrial Science, The University of Tokyo, 4-6-1 Komaba, Meguro-ku, Tokyo 153-8505, Japan

Tel : +81-3-5452-6898,

E-mail : kaseda@iis.u-tokyo.ac.jp

Recently self-heat recuperation technology based on exergy recuperative principle has been studied. This technology is able to recirculate all the process heat in a system by making temperature difference through vapor compression. It is mainly used for separation processes. However, the current self-heat recuperation system has two drawbacks. One is that it cannot be used for liquid and solid, and the other is it cannot be used for micro fluid processes. Therefore a novel self-heat recuperation system using electrocaloric effect (ECE) as a heat circulator is proposed. In this system, ECE is used in place of vapor compression. ECE is a phenomenon in which ferroelectric material shows temperature and entropy change adiabatically under electric field. Other studies concluded that ferroelectric material shows the biggest temperature change near its curie temperature by ECE, thus the simulation was conducted near this temperature. The performance of this system was evaluated using T-S diagram and the ratio of circulated heat (Q) to the electric work (W).

In the simulation, barium titanate was used as a ferroelectric material. Temperature and entropy of barium titanate was calculated under two applied electric fields, 0V/m and 1MV/m. The results showed that near curie temperature of about 397K, temperature change between two conditions was maximized to 2.5K. The ratio of heat to work when its minimum temperature of cycle is 385K and maximum temperature is 395K is 60.94 (Q/W).

From this result, it is found the proposed self-heat recuperation system using ECE has high efficiency, thus it has a potential for energy saving for thermal processes.