Engineering Alternative and non-Natural Routes to Aromatic Alcohols

Brian Thompson, Shawn Pugh, Rebekah McKenna, and David R. Nielsen

Aromatic alcohols represent a useful class of target molecules with a diversity of applications including as monomers, building-block compounds, antioxidants, and flavors and fragrances.  Although traditionally derived from petroleum feedstocks and/or directly from plant biomass, recent efforts to engineer microorganisms for their direct biosynthesis from renewable biomass sugars show promise for improving the sustainability and tractability of the approach.  Specific and recent examples include the engineering of Escherichia coli to produce phenol, catechol, and 2-phenylethanol directly from glucose.  Established routes, however, suffer from several inherent limitations.  In the case of phenol, for example, which is derived from endogenous tyrosine via tyrosine phenol lyase (TPL), feedback inhibition and equilibrium limitations at TPL reduce its activity as well as flux through the overall pathway. Conventional approaches for the biosynthesis of catechol (which typically has been derived from 3-dehydroshikimate via a two-step pathway), meanwhile, rely upon the creation of multiple auxotrophies that then necessitate the supplementation of several expensive amino acids and vitamins into the culture media.  To address these and other limitations, alternative and non-natural biosynthetic pathways leading to the same three aromatic alcohol products have been devised and comparatively evaluated.  Said novel routes stem from alternative endogenous precursors and incorporate dissimilar heterologous chemistries to circumvent notable limitations of each established pathway.   For example, in the case of catechol, the proposed alternative route stems instead from endogenous p-hydroxybenzoate and proceeds to catechol in two steps via the intermediate protocatechuate.  By this new approach, catechol titers as high as 631 ± 37 mg/L have so far been reached with limited adjustment to the composition of the minimal media.   Phenol bioproduction, meanwhile, is being explored via three distinct putative pathways.  Lastly, as an alternative to the Ehrlich pathway, a novel route to 2-phenylethanol has also been engineered.  To do so, the previously developed styrene pathway was systematically extended to also include key steps involved in styrene biodegradation.  Maximum titers thus far achieved by this alternative route are currently 460 ± 50 mg/L.  In this study, the most recent findings from these proposed alternative pathways will be compared and contrasted with those of established routes to provide further insight regarding the best practices for aromatic alcohol biosynthesis.