## Modeling the Drying Kinetics of Carrot Particles in a Closed Chamber Following Forced Convection and Electric Field Treatment

## A.B. Jemaia, L. Khezamib, M. HadjKalia, and E. Vorobievc

- King Saud University, College of Engineering, Chemical Engineering Dept., P.O. Box 800 Riyadh 11421- KSA.

- Al Imam Mohammed Ibn Saud University, Faculty of Sciences-Department of Chemistry. P.O.Box: 90950 Riyadh: 11623 Kingdom of Saudi Arabia.

- University of Technology of Compiegne, Chemical & Industrial Process Engineering Department

The impact of forced convection and Pulsed Electric Field pretreatment on the drying kinetics of grating type carrot particles, placed in a closed chamber, have been studied based on continuous relative humidity measurements (ΔΦ) of circulating air.  The particles were placed in a laboratory-scale closed chamber (30g particles capacity) which can be used for the subsequent pressing operation and drying air circulation.  The kinetics of moisture content loss was indirectly deduced from a normalized form of the relative humidity difference between the entering and leaving air streams. Using two thermo-hygrometers, the continuous measurements of relative humidity difference between entering and exiting air were used in a normalized form to study the kinetics of drying for various air speed. The effect of an electric field pre-treatment of the particles was also illustrated using this experimental setup. About 20% reduction in drying time could be achieved following PEF pretreatment.

Keywords: Agro-food particles processing, Relative humidity, Modeling Drying kinetics, Forced convection, Electric fields.