Quantitative Phenotyping of Tiling Synapses in C. elegans

C. elegans, a model organism which helped scientists elucidate the mechanisms of programmed cell death and RNAi, also enables the study of neurological development in a simple and very well-stereotyped model. The ease of culture and fecundity as well as the transparent cuticle of these worms lends itself to genetic screens for desired phenotypic traits. By searching for unique phenotypes among mutagenized populations, genetic causes of relevant phenotypes can be investigated. However, traditional screening is a time intensive and tedious task. In addition, many subtle phenotypes are difficult for experts to pick out, such that many potentially intriguing and biologically significant phenotypes are overlooked.

Correct neurological development in worms relies on the proper patterning of synapses in motor neurons. This requires contact-dependent juxtacrine signaling and an array of receptors and small molecules, such as semaphorins and their receptors, which are still poorly defined. When synaptic domains of two different motor neurons overlap, the muscle that is the target of the motor neurons may receive misinformation about which segment of the muscle to innervate, which can result in subtle differences in movement. Using a transgenic strain with fluorescent markers in the synapses of two partially overlapping motor neurons in C. elegans, defects in this proper patterning, called “tiling”, can be found, allowing the discovery of genetic controllers of neurological development. Because of the high homology (~40%) between C. elegans’ genome and the human genome, errors in developmental processes, such as synaptic tiling in worms, can give insight into diseases of the central nervous system in humans.

Through the use of microfluidics and computer vision techniques, an automated, high-throughput screening system has been developed for identifying tiling mutants. Worms are imaged on a microfluidic platform designed for imaging large volumes of worms efficiently. By training a support vector machine (SVM) to identify synapses in each of two tiling motor neurons, decisions about each worm’s phenotype can be made autonomously. Using these decisions, a mixed population of worms can be sorted on-chip. Taken together, this offers a much more efficient genetic screen, on the order of 100 times faster than traditional screening methods.  By using quantitative instead of qualitative methods for identification of tiling mutants, previously unknown and subtle phenotypes can be discovered which will shed light on neurological development in humans as well as worms.