Session: 15D19 Micro-Scale Technologies

Microfluidic Platform for Combinatorial Screening of Chemicals in Caenorhabditis elegans

Guillaume Aubry and Hang Lu

guillaume.aubry@chbe.gatech.edu, hang.lu@gatech.edu

School of Chemical & Biomolecular Engineering, Georgia Institute of Technology, Atlanta GA, USA

The nematode Caernorhabditis elegans (C. elegans) is a well-known model organism for research on behavior and development. In particular, its chemosensory system has been widely investigated to understand chemosensation, chemotaxis, and how a stimulus is processed to trigger a specific behavior. Many studies have focused on one specific molecule and its chemosensory neuron associated but, to approach situations closer to real environment, multiple stimuli must be considered. However, investigating the effects of multiple chemical stimuli reveals to be challenging, not only because of the large panel of molecules that the nematode senses, but also because the chemicals work in a concentration-dependent manner and may present synergistic effects. Conventional methods are not adapted to fulfill this task, because of the high consumption of reagents, and therefore a new technology is required to enable such a project.

The aim of this work is to develop a platform allowing for combinatorial screening of pheromones in C. elegans. Using droplet microfluidics and on-chip valve technology, we designed a device that can create different mixtures of reagents and stimulate worms. The advantages for using droplets are to minimize the consumption of reagents and to avoid cross-contamination. Using on-chip valves actuated in a specific sequence to deliver droplets on demand, we demonstrated the creation of precise mixtures for a set of three reagents. The concentrations could be tuned as well by achieving on-chip serial dilutions. To demonstrate capability of this device as a screening platform of chemicals, we studied male attraction to pheromones from a crude extract of culture media. The worms were trapped in 1 mm large droplets stored in chambers and imaged simultaneously at low resolution. Their movement was analyzed to quantify attraction in function of pheromone concentration.

We envision this platform to be determinant in studying competitive effects of attractants and repellants in C. elegans behaviors, as well as identifying new synergistic combinations. Future developments include integrating additional environmental cues such as thermal or mechanical stimuli to investigate the processing of multiple stimuli to trigger a specific behavior.