Nate Gomes (nategomes@my.uri.edu)

AIChE - URI Student Chapter

Abstract for AIChE National Student Conference

Fall 2014

Microstructural Analysis of Colloidal Silver Precipitates

This investigation focuses on the characterization of colloidal silver particles precipitated from an ammonium nitrate solution by neutralization of silver nitrate using ammonium hydroxide.  Pellets of silver were reacted with nitric acid to form silver nitrate solution and allowed to cool and dry.  Ammonium hydroxide was added to the silver nitrate crystals, forming an ammonium nitrate solution with nanoscopic silver colloids dispersed throughout the liquid phase.  These preliminary reaction steps were necessary to produce the desired final product, i.e. a fine silver precipitate having the desired particle size and particle size distribution.  These precipitated powders are routinely made in industry for use in several high performance electrical applications.  We are investigating methods for fine-tuning the particle size distribution of the silver powders produced by this precipitation process.  We believe if we can form uniform size silver colloids, we may be able to use these as seed crystals to grow the desired silver precipitates with the desired particle size and size distribution.

Silver colloids were isolated and imaged for microstructural and chemical analysis using Scanning Electron Microscopy and EDS (Energy-Dispersive analysis of X-rays). Elemental analysis of the colloids indicated they were largely silver and the resulting silver colloids appeared to form a monodisperse with a mean particle size of one micrometer (in diameter).  This confirms our hypothesis that the uniformity of the silver colloids was sufficient to act as a template for subsequent growth of the silver particles with the desired particle size and particle size distribution.