Full Spectrum Phosphor Based LEDs

Jannatul Ferdaous, City College of New York

Mentor: Partha Dutta, Rensselaer Polytechnic Institute

Abstract

Replicating sunlight for indoor lighting has been one of the many goals of researchers and companies since centuries. The goal of this research is to determine the ratio of a mixture of three to four different colors of phosphors, produced from rare Earth materials, which closely replicates the sunlight or produces a light which has a full spectrum of the visible range when excited by a blue UV light. After a ratio of phosphor powders was determined to give a high CRI as the sunlight, a multi-phase phosphor powder was to be produced from the rare Earth materials, consisting of the same ratio which was determined to best replicate the sunlight. The purpose is to produce a single phosphor powder which alone can replicate the sunlight instead of having to mix several phosphor powders to attain a high CRI. Several ratios of phosphors have been determined which emit light of CRI above 90. However, reaction of the Earth materials to produce one multi-phase powder of that ratio has not been successfully produced yet.