Role of permeate flux and specific membrane-foulant-solute affinity interactions (∆Gslm) in transport of trace organic solutes through fouled nanofiltration (NF) membranes

T.O. Mahlangua,b,*, K.V.K.M. Schouttetenb, A. D’Haeseb, J.M. Thwalac, B.B. Mambaa, A.R.D. Verliefdeb

a University of South Africa, Nanotechnology and Water Sustainability Research Unit, College of Engineering, Science and Technology, Florida Science Campus, Roodepoort

b Ghent University, Faculty of Bioscience Engineering, Particle and Interfacial Technology Group (PaInT), Coupure Links 653, B–9000 Ghent, Belgium

c University of Swaziland, Faculty of Science, Department of Chemistry, Kwaluseni Campus Private Bag 4, Kwaluseni, M201, Swaziland

* Corresponding author; E–mail: mahlanguoranso@yahoo.com, Tel: +32 9 264 60 01, Fax: +32 9 264 62 42

Abstract

In the literature, sieving effects, charge interactions and adsorptive/hydrophobic interactions have been named as major mechanisms influencing the rejection of trace organic compounds (TOrCs) or pharmaceuticals. However, dominant mechanisms influencing TOrC rejection by fouled membranes are not yet fully understood. Most studies have attributed the decline in rejection to cake-enhanced concentration polarisation (CECP) effects and have overlooked the influence of flux decline and attractive membrane-TOrC interactions. Therefore, the aim of this study was to investigate the role of various factors in TOrC rejection for nanofiltration (NF) membranes fouled in cross-flow filtration. To further gain insight into the role of affinity interactions on TOrC rejection, membrane-solute interaction energies (∆Gslm) were computed from the contact angle measurements based on the Lifshitz-van der Waals/acid-base approach. A novel fouling approach (termed sequential fouling) is used to investigate CECP effects for different fouling layers. For a virgin membrane, TOrCs are rejected through sieving mechanisms and repulsive non-electrostatic membrane-TOrC interactions. Upon fouling (especially in the presence of calcium), there was an improvement in the rejection of some TOrCs (solute permeation decreased) due to an improvement in sieving effects. The rejection of some TOrCs decreased over time (solute permeation increased) but the rejection values were close to those of the virgin membrane at similar water fluxes showing the influence of flux. When solute permeation increased as a result of fouling, TOrC transport was mainly influenced by non-electrostatic membrane-TOrC affinity interactions and CECP effects (as evidenced by novel sequential fouling experiments).

Keywords: pharmaceuticals; membrane-solute affinity; nanofiltration