Modeling gas sorption in thermally rearranged polymers:

a thermodynamic approach

Michele Galizia, Kevin A. Stevens, Zachary P. Smith, Donald R. Paul, Benny D. Freeman

J.McKetta Department of Chemical Engineering, 200 E. Dean Keeton Street, 78712 Austin,

and Center for Energy and Environmental Resources, 10100 Burnet Road, 78758 Austin,

The University of Texas at Austin, Texas (USA)

Thermally rearranged (TR) polymers have attracted increasing attention in natural gas sweetening [1]. These materials, first reported by Park et al. [2], offer an unusual combination of high gas permeability and selectivity, which, in some cases surpass the upper bound defined by Robeson in 2008 [3]. TR polymers can be prepared via thermal rearrangement of solvent-soluble polyimides to form insoluble poly-benzoxazoles.

In this work, hydrogen, nitrogen, methane and carbon dioxide sorption isotherms in three TR samples, obtained via thermal conversion of HAB-6FDA polyimide, were theoretically interpreted using the Non-Equilibrium Lattice Fluid (NELF) model [4]. Since PVT data in the rubbery region are not accessible for HAB-6FDA and its thermally rearranged analogs, the polymer characteristic lattice fluid parameters, T*, p* and *, were estimated using a collection of mixture data at multiple temperatures [5]. The model outcomes show good agreement with the experimental sorption isotherms for both the pristine polyimide and the corresponding thermally rearranged samples. Interestingly, the model confirms by a completely independent perspective that the increase in gas solubility in TR samples is due mainly to entropic factors, i.e. the increase of excess free volume upon thermal rearrangement. This result is fully consistent with the experimental evidence [1,2]. Finally, the model results indicate that TR samples have, in the presence of carbon dioxide, greater dimensional stability relative to HAB-6FDA polyimide.

[1] D.F. Sanders et al., J. Membr. Sci. 2012 409-410 232-241

[2] H.B. Park et al., Science 2007 318 254-258

[3] L.M. Robeson, J. Membr. Sci. 2008 320 390-400

[4] F. Doghieri et al., Macromolecules 1996 29 7885-7896

[5] M. Galizia et al., J. Membr. Sci. 2012 405-406 201-211