Room-Temperature Synthesis of Functionalized Two-Dimensional Metal-Organic Frameworks for Enhanced CO2/CH4 Separation

Jie Zha, and Xueyi Zhang*

Department of Chemical Engineering, The Pennsylvania State University, University Park 16802, U.S.A.

E-mail: xuz32@psu.edu

Abstract

Removal of CO2 from raw natural gas is an essential process prior to the actual utilization of nature gas. Mixed-matrix membranes (MMMs) have showed advantages in CO2/CH4 separation, but great challenges exist in controlling the morphology and functionality of filler materials, which plays a key role in gas permeation and compatibility between filler materials and polymer matrix. Metal-organic frameworks (MOFs), due to their versatile synthetic procedures, exhibit tunable structures and chemical functionalities, and thus are good candidate for preparing MMMs. Herein, we demonstrated a one-step room-temperature synthesis of 2D MOFs based on Cu (II) paddle-wheel units with mixed ligands, where the intrinsically anisotropic building blocks led to the anisotropic growth of 2D MOF nanoparticles. This synthetic method was also successfully extended to prepare functionalized 2D MOFs by selecting ligands with different functional groups, with the thickness of the thinnest particles around 3 nm. To illustrate the advantage of using 2D MOFs, the synthesized 2D MOFs were incorporated into polysulfone, a commercial polymer for CO2/CH4 separation, and the resultant mixed-matrix membranes (MMMs) exhibited improved CO2/CH4 separation performance with CO2/CH4 ideal selectivity increased by approximately 400% at 5 wt% loading of amino-functionalized 2D MOFs. Because the functionality of 2D MOFs can be readily tuned based on the selection of ligands, the presented strategy to prepare MMMs can be further extended to other environment-related separation processes.