Interfacial Interactions of Lignin Dimers with Synthetic Lipid Bilayer

Mahsa Moradipour1, Audrey Fetsko1, Poorya Kamali2, Shardrack O. Asare2,

Bert C. Lynn2, Stephen E. Rankin1, Barbara L. Knutson1

1Department of Chemical Engineering and Materials Science, University of Kentucky

2Department of Chemistry, University of Kentucky

Lignin is an organic substance present in cells, cell walls, and between the cells of all vascular plants. While comprised of only three monolignols (aromatic alcohols), it has a complex structure due to its bond types.  Due to the heterogeneity of the lignin structure, the goal of deconstructing lignin to commercially viable small molecules for use as chemicals or advanced materials has been elusive.  One potential application of lignin derived molecules is antimicrobials or antifouling coatings.  This application is based on the known inhibitory nature of deconstructed lignin components to microorganisms.

In this work, the interaction of three β-O-4 dimers of coniferyl alcohol (GG lignin dimer) with synthetic lipid bilayers was studied: a commercially available compound with no alcohol tail, the “natural” dimer containing a hydroxyl tail, and a benzyl-modified dimer. Small lignin molecules are thought to act as antimicrobials by interacting with cell membranes, represented here as model phospholipid bilayers. A quartz crystal microbalance (QCM) was used to study the amount of dimer uptake on supported lipid bilayers and compare effect of dimers on the properties of the bilayer.  Corresponding differential scanning calorimetry measurements (DSC) using lipid vesicles were used to more directly study the change in bilayer phase behavior due to disruption of the bilayer. These measurements suggest that the commercial GG and the benzyl-modified GG enter the hydrophobic region of the bilayer, with similar partition coefficients (logKm/w) of 2.9 and 3.0, respectively. However, significantly less of natural dimer entered the lipid bilayer (logKm/w = 2.10). These results indicate that minor differences in structure in lignin-based derivatives have a significant impact on their interaction with lipid bilayers and that the techniques used in this study will provide fundamental insight into interactions for different lignin monomers, dimers, and trimers.

- This project is supported by NSF EPSCoR Track-2 RII, Award No. OIA1632854, and National Science Foundation REU Program, University of Kentucky (grant no. 1460486